# Electronic  Crane  Scale

# User’s Guide

Table of Contents

- Overview

- Features & Specifications

- Functions of Key Pad & Indicators

- Basic Operations

  - Power ON

  - Initial on Zeroing

  - Manual Addition

  - Display of Total Weight

  - Set Parameters

  - Weighing

  - Power OFF

- Battery Charging

- Scale care

- Calibration

- Dimensioned Drawing of Scale

- Instruction of Infrared Control

10.Optional Accessories

- Overview

This series crane scale is a high-tech scale. The digital crane scale employs both the advanced hardware and sophisticated software. It uses AT-89 series single-chip micro-processor and highly precision A/D converter. In addition, the crane scale is equipped with specially designed compensating circuitry for rocking loads. With its quick stabilization and strong interference-rejection, it can be widely used in weighing applications in commerce, industrial and mining industries, and warehousing wharf, etc.

- Features  & Specifications

| Accuracy Class | III |
| --- | --- |
| Initial on Zero Range | Selectable 0%、5%、10%、20%、50%、100% F.S |
| Time to Stable Static Reading | ≤10 seconds |
| Maximum Safe Load | 150% F.S. |
| Display Screen | Super bright 1.2″5-digit LED display |
| Operating Temperature | -10 °C ~ +40°C |
| Operating Humidity | ≤85%RH |
| LOB Indicator | When the power is low, the indicator will be on |
| Auto-OFF | The scale will shut off automatically once the power is severely low. |
| Recharging Protective Circuitry | To prolong the performing life of the battery |
| Cast Aluminum Housing | Rust protection, antistatic & light weight |

| Max. Capacity (kg) | Mini. Weight(kg) | Measured Division | Selectable Division |
| --- | --- | --- | --- |
| 600 | 4 | 0.2 | 0.1 |
| 1000 | 10 | 0.5 | 0.2 |
| 2000 | 20 | 1 | 0.5 |
| 3000 | 20 | 1 | 0.5 |
| 5000 | 40 | 2 | 1 |
| 10000 | 100 | 5 | 2 |
| 15000 | 100 | 5 | 2 |

- Functions of  Key Pad & Indicators

Presented below is the scale panel:

As presented on the panel,                          ,                          are operating keys, of

whose functions are in the following:

  - –Use this key to add the load weight.  It is only usable when the scale is in manual mode of operation.

  - –Press this key to keep the current displaying weight and the            indicator will be on.  Press the key again to turn off temporarily holding function. In the value input, press this key to add 1 and circulate in the 0 to 9.

  - – Press this key to tare and the             indicator will be on.  Press the key again to turn off temporarily tare function.

  - – Press this key to zero the current displaying value.

  - LB/KG- When the red light on it means the unit is on KG. When the light turn green, the unit is on LB

- Basic Operations

  - Power On

Turn on the power switch at the back of scale to the ON position.  The scale will display the version number and then the maximum weight capacity individually for one second. Then the nixie light will display “99999”,”88888”…….”00000” in turn. Then scale will undergo self-check of reset sequence and indicators with LED indicators on and off alternatively and finally come into weighing mode.

  - Initial on zeroing

After power on, the scale enters into weighing mode and then displays “0”kg. If it is not “0’kg, it’s normally caused by overloading the permitted value of the initial on zero range. In this case, please switch off the scale and re-start it and choose the desired parameter.  (Please see initial on zero Range below.)

  - Manual Addition

When the displaying value is over zero, with weight on the scale, pressing               key, the current weighing value will be added to the total weight if with valid parameter. (Please see 4.5 Set Parameters.) The scale will show “bXXXX” for about one second.  Here, “b” means the total number of loads and “XXXX” indicates the total number of adding.

Note: No matter manual add or atuo-add, please zero the displaying value before another manual adding. Here, the zeroing range includes 20 division value of the capacity. When the number of adding reaches 9999, it will display FULL which indicates full memory. In this case, please clear the memory according to “4.7 Clear” in the following.

  - Display of Total Weight

When the displaying value is within the zeroing range, press key             to see the total number of adding and the total weight of loads. It will firstly display the total number of loads. For example, the scale shows “b0327” indicating 327 loads weighed so far.  Press                 again, it will display the four left-hand number of the added value. For instance, “H 108” indicates the left-hand number of the total weight is “108”.  Press               for the third times, it will show the four right-hand number of the total weight.  For your reference, “ L 2896” indicates the right-hand number of the total weight is “2896”. Accordingly, the total weight of the loads is “1082896 kg”.

  - Set Parameters

Press [             ] during self-check sequence from “99999” to “00000”

| Function | Operation | Display | Remarks |
| --- | --- | --- | --- |
| Division | Press [ ] during self- check sequence from “99999” to “00000” | dS 1 | 0: Measured Division 1: Selectable Division Press [ ]to change. |
| Sets buzzer | Press [ ] | bEE 1 | 0: Disable 1: Enable Press [ ]to change. |
| Zero point drift | Press [ ] | to 1 | 0: Close 1: Activate Press [ ]to change. |
| switch off the scale mode | Press [ ] | oFF 2 | 0: The scale will keep displaying. 1: The displaying window will go into sleeping mode with stable weighing value for 30 minutes. It will be activated when the displaying value is unstable or press the[ ] key . 2: The scale will keep working for 30 minutes with stable weighing value displayed and then power-off automatically. Press [ ]to change. |
| Addition condition | Press [ ] | AS 1 | 0: Adding allowed when unstable 1: Adding not allowed when unstable Press [ ]to change. |
| Sets units | Press [ ] | Unt 0 | 0: kg 1: lb Press [ ]to change. |
| Initial on zero range Setup | Press [ ] | o 20 | percent of capacity Press [ ]to change. |
| Clear | Press [ ] | CLEAr | Press [ ]save clear Operation then quit. Press [ ] to quit without save. |

  - Weighing

During the weighing process, the weight is ready to be recorded when the stable weight result is shown and the STAB indicator is on.  If there is tare involved, such as rope or container, weigh them first.  Press             to apply tare.  The display should now be “0.0kg”, the net weight.  With no change to the tare, the loads weighed afterwards will be their net weights until the change of tare value or the scale is turned off.

  - Power Off

After the weighing operation, turn off the scale by using the power switch on the back of the scale.

- Battery Charging

  - It adopts maintenance-free rechargeable battery (6V/4Ah) and smart charger (DC6V/1Ah).

  - Then LOB indicator will be on if there is low power. The battery needs to be charged now. The scale, however, can continue to operate for about 5hours without charging.

  - After the LOB indicator is ON for several hours without charging or there is no more power in the battery, the scale will be powered off automatically.  It will take about 5 hours to fully charge the battery.

  - After battery charging, if there is no display, it indicates that either there is difficulty in battery charging or a damaged battery.  Please contact supplier for repair or replacement.

  - To prolong the battery life, we suggest you charge the battery no less than once per week. If the scale needs not to be charged for long time, you should charge it no less than once per week. To avoid any damage to the battery, please charge it correctly. Additionally, we suggest you switch off the scale to short the charging time.

- Scale Care

  - Do not overload the scale to prevent the damage to the force transducer inside the scale. It will alarm you with buzzer and display “OVErl” if overloaded.

  - If display “Err01”after power on the scale, indicates that  it will influence the accuracy or the important parameters lost.

  - Please check the scale frequently to insure the safe use. The basic checking list is:

6.3.1 Whether the screw has come loose

6.3.2 Whether the hook or shackle or any other component is cracked or distorted

6.3.3 If the mechanical components showing sign of metal fatigue or defects

6.3.4 Whether the pin is off or the components are lost.

If there is any sign of the mentioned condition, please send the scale or components back to the manufacturer for repair or exchange before using the scale again.

  - Avoid any impact to the display panel.

  - Do not use the scale under wet or heavily polluted conditions.

7. Calibration

| Steps | Operation | Display | Remarks |
| --- | --- | --- | --- |
| 1 | Hold on to power on the scale | 00000 | 1st step of calibration. Password is required. |
| 2 | Press to select the desired number, press to shift digit | ****** | ****** is the password |
| 3 | After entering password, press | SPEN | There should be no load on the scale |
| 4 | Press | 9500 | 9500 is the encode at zeroing loading |
| 5 | Press | 00000 | Wait until the codes become stabilized and press [ADD/INPUT] |
| 6 | Press to change to the desired number for the standard weight, press to shift digits | 03000 | 03000 is the standard weight of 3000kg to be loaded. |
| 7 | After the standard weight is loaded, press | 77683 | 77683 is the encode number of standard weight - 3000kg. |
| 8 | Press | 3000 | Calibration is completed. The scale is in weighing mode. The current weight load is displayed. |

Note: 1.Please make sure to wait until the encode is stabilized then press                 . Or it will lead to incorrect weighing results.

2. Do not calibrate the scale without standard weight.

- Dimensioned Drawing of Scale

| Model | B | G | H | L | L1 | L2 | D1 | Ф2 | Фd | ФF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XZ-06JJE | 120 | 163 | 198 | 420 | 190 | 33 | 40 | 47 | 21 | 76 |
| XZ-1JJE | 120 | 163 | 198 | 420 | 190 | 33 | 40 | 47 | 21 | 76 |
| XZ-2 JJE | 120 | 163 | 198 | 420 | 190 | 33 | 50 | 47 | 21 | 76 |
| XZ-3 JJE | 120 | 163 | 198 | 462 | 196 | 40 | 50 | 50 | 23 | 76 |
| XZ-5 JJE | 120 | 163 | 198 | 511 | 204 | 47 | 52 | 60 | 26 | 80 |
| XZ-10 JJE | 135 | 181 | 206 | 589 | 239 | 66 | 52 | 70 | 36 | 90 |
| XZ-15 JJE | 135 | 181 | 206 | 727 | 257 | 77 | 60 | 85 | 47 | 135 |

- Instruction of Infrared Controller

Remote Control & Usage is the same with the scale body button.

(the controller powered by AAA battery)

10. Optional Accessories

The scale could match one kind of our palm indicator after assembled the wireless A/D.

★  The products are subject to change without notice.