# OCS-SZ Series User’s Guide

# Crane Scale with Wireless Indicator

## Table of Contents

1 Product Features…………………………………………………………………………………3

2 Key Technical Specification and Basic Parameters……………………………………………..4

3 Model Specifications…………………………………………………………………………….5

4 Indicator Panel…………………………………………………………………………………...5

5 Key Pad Functions……………………………………………………………………………….5

6 Introduction Mode of Operation…………………………………………………………………6

7 Basic Operations…………………………………………………………………………………7

(1) Power ON……………………………………………………………………………..........7

0/ZERO………………………………………………………………………………………..8

1/AUTO……………………………………………………………………………………….9

2/ADD………………………………………………………………………………………..10

Manual Add……………………………………………………………………………….10

Auto Add………………………………………………………………………………….11

3/Add…………………………………………………………………………………………11

4/Print H……………………………………………………………………………………...12

5/Order……………………………………………………………………………………….13

6/Division……………………………………………………………………………….........14

7/Tare…………………………………………………………………………………………15

8/Subtract……………………………………………………………………………………..16

9/Feed…………………………………………………………………………………………17

Search…………………………………………………………………………………………17

Backligh/Confirm……………………………………………………………………………..18

SET……………………………………………………………………………………………18

[1] Clear……………………………………………………………………………………….19

[2] Summarize………………………………………………………………………………....19

- Printing Per Sequence Number………………………………………………….20

- Printing Per Order Number……………………………………………………...20

- Printing Per Weighing Date……………………………………………………..21

[3] Total Clear…………………………………………………………………………………22

[4] Set Date…………………………………………………………………………………….23

[5] Set Time……………………………………………………………………………………23

[6] Zero Range…………………………………………………………………………………24

[7] Others……………………………………………………………………………………....25

[1] Set Weight Limit…………………………………………………………………….25

[2] Calibration…………………………………………………………………………...26

[3] Change Password……………………………………………………………………29

[4] Zero-point Track Range……………………………………………………………..29

[5] Filter Setup…………………………………………………………………………..30

[6] Auto Print Setup……………………………………………………………………..30

[7] Printer Setup…………………………………………………………………………31

[8] Indicator Exchange…………………………………………………………………..31

8 Battery Charging Method…………………………………………………………………….32

9 Precautions…………………………………………………………………………………...32

10 Trouble-shooting……………………………………………………………………………33

11 After Sale Service…………………………………………………………………………...34

12 Packaging List………………………………………………………………………………34.

1. Product Features

OCS-SZ digital wireless crane scale is composed of two parts, a scale and a force indicator.  The scale uses a patented high precision resistant-strain transducer and employs a reliable force transfer structure.  Combined with the multi-function intelligent indicator, the weighing system is very capable for application in specified range of weighing operation.

The key features of the force indicator are:

- Compact and light weight for portable operation

- Backlighting equipped LCD display for great visibility under low light operation environment.

- Build-in calendar and clock

- Build-in Epson micro printer that can print up to 9999 set of weighing data according to the measurement date, order or weighing sequence

- Large memory space to store up to 2,900 lines of data.

- Battery power level monitor for scale and indicator

- Overload warning for safe operation

- Auto-off when the scale remains inactive for over 2 hours

### Indicator and OCS Crane Scale Dimensions

- Force Indicator

Fuse , Serial Output , Charger input, handle , LCD display, printer, power switch, antenna

- Scale

hanging hook, antenna , scale body , load transducer, A/D processor , hook ,battery housing

- Model Type

| Model Type | Range t | Division kg | dimension (mm) |  |  |  |  |  | weight kg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A | B | C | D | E | H |  |
| OCS-SZ-1C | 1 | 0.5 | 76 | 105 | 34 | 190 | 47 | 460 | 15 |
| OCS-SZ-2C | 2 | 1 | 76 | 105 | 34 | 190 | 47 | 460 | 15 |
| OCS-SZ-3C | 3 | 1 | 76 | 105 | 40 | 190 | 50 | 486 | 15 |
| OCS-SZ-5C | 5 | 2 | 80 | 120 | 48 | 200 | 60 | 560 | 23 |
| OCS-SZ-10C | 10 | 5 | 90 | 128 | 52 | 216 | 70 | 630 | 36 |
| OCS-SZ-15C | 15 | 5 | 135 | 141 | 60 | 216 | 85 | 897 | 52 |
| OCS-SZ-20C | 20 | 10 | 150 | 148 | 73 | 240 | 90 | 948 | 103 |

2.  Key Technical Specification and Basic Parameters

- Accuracy Class:	GB11883-89 (International R76 equivalent)

- Power Source: 6V/4AH Nd-H battery

- Radio Frequency: 430MHz (8 frequency band)

- Load Cell Activation Power: DC 5V±5%

- Operation Temperature: 10°~50°C and 0°~40°C for scale and indicator, respectively

- Radio Transmission distance: >200m unobstructed

- Model Specifications

| Model Type | Maximum Capacity（kg） | Division (kg) | Weight Limit（kg） |
| --- | --- | --- | --- |
| OCS-SZ-1 | 1000 | 0.5 | 1004.5 |
| OCS-SZ-2 | 2000 | 1 | 2009 |
| OCS-SZ-3 | 3000 | 1 | 3009 |
| OCS-SZ-5 | 5000 | 2 | 5018 |
| OCS-SZ-10 | 10000 | 5 | 10045 |
| OCS-SZ-15 | 15000 | 5 | 15045 |
| OCS-SZ-20 | 20000 | 10 | 20090 |
| OCS-SZ-30 | 30000 | 10 | 30090 |
| OCS-SZ-40 | 40000 | 20 | 40180 |
| OCS-SZ-50 | 50000 | 20 | 50180 |

- Indicator Panel

- Key Pad Functions

| Keys | Function descriptions |
| --- | --- |
| 0~9 | Numerical keys, they can also be used with other function keys |
| ZERO | Zero the current weight display. |
| Auto | Start or End auto storing or printing function. |
| Add | Add current stable weighing data to the internal memory, including parameters, such as sequence number, index, date and time, etc. |
| Total | Show the total weighing number and total weight |
| Print H | Print the header for the data sheet |
| Order | Change the current order number (0000~9999) |
| Division | Set the division number or the minimum display variable number |
| TARE | Set the known tare number (0000.0 ~9999.9) |
| Subtract | This function is mainly used for milling or molding application to indicate the amount of weight subtracted. |
| Feed | Forward the print paper for four lines without printing |
| Search | Search the existing weighing data |
| Setup | Set the system index |
| Backlight/Confirm | Turn on the backlighting when the display is for weight or time. Confirm for others. |
| Print | Print the weighing data (two types of printing method) |
| Off/Cancel | Turn off the indicator or cancel specified operating steps |
| On | Turn on power supply to the system |

- Introduction -  Mode of operation

Immediately after the indicator is turned on, it will go through self-check sequence, including the version of the operating interface, measurement capacity limit, then the numbers from 999999, 88888, 777777, ….to 111111.  There will be 0.5 second interval between each set of numbers.

After the completion of self-check sequence, the time mode will be displayed as Fig. 1.  If there is weighing signal detected, the display will change into Weigh mode from time mode as shown in Fig. 2. Otherwise the display will remain in Time mode.

09：15：45

As shown in Fig. 1, the battery power level indicator is located in the upper left corner of the LCD display.  Four-bar indicates the battery is at full charged full, one-bar means the battery power is low.  In that case, the indicator needs to be charged to prevent it from data lost.  09:15:45 is the current time.

Auto

NO.1

0005

0．0 kg

NO.1

0005

In Fig.2, RECEIVE  indicates there is weighing signal detected by the indicator and the indicator is in weighing mode; AUTO means that the indicator is in automatic store and print states; STABLE means the measured weight has been at stable condition; ZERO indicates that there is 0 weight; the number under Sequence is the current weighing number (as shown as 005 in Fig. 2).  Scale Power Low indicates that the scale battery needs to be changed so to maintain proper signal transmission.

- Basic Operations

The basic operations for the indicator are described below.

  - Power ON  - For simple weighing operation, it can be done by following operating sequence:

    - For initial use, connect the battery of the crane scale:

1) open the battery housing using a flat-tip screw driver and connect the battery to the power cable;

2) Switch the power to ON position.  The scale is powered on when a long beep is heard.

    - 30 seconds after the scale is powered on, turn on the indicator power. Press ON key on the indicator.  The display will show the software version first (such as “V1.0” as shown in figure below) and then the weight value limit (as shown as “5018” in this case). After that, the display will cycle from “999999” all the way down to “111111”.  If there is no weight signal received from the scale, the display will remain in Time mode as shown in Fig. 1.  If there is weight signal received, the indicator will change to WEIGH mode (Fig.2).

    - After the load is craned up through the scale and STAB is continuously shown on the upper right corner of the display, the weight value displayed is the weight of the load.

V1.3

5018

999999

888888

999999 to111111conter down

111111

09：58：41

NO.1

0001

0 kg

If signal detected

  - ZERO

The indicator should show “0.0” under non load condition. In case there is residual value displayed under zero loading, such as 12kg as shown in figure below; it has to be zeroed before weighing operation.  Press ZERO to zero it out.  The weight display should be 0kg and ZERO is lit. Zeroing function is only effective when the residual display is less than

20%  F.S. of  the scale.

NO.1

0001

12 kg

NO.1

0001

0 kg

Press ZERO

  - AUTO

In WEIGH mode, press Auto key and AUTO is lit.  Now the system is in automatic mode of operation for date storage and print.  Press Auto key again, AUTO indicator is off.  In this mode, the system is operating in manual mode.  Any weight data has to be manually totaled to be saved.

NO.1

0001

0 kg

Aut  Auto

NO.1

0001

0 kg

NO.1

0001

0 kg

Repress AUTO

Press AUTO

With AUTO shown on the display, each weighing result is automatically saved in the data memory after preset time delay (see Auto-print stable time setup).  The result is also being printed with its weighing sequence number.

This AUTO process is illustrated in the diagram below:

NO.1

0001

0 kg

NO.1

0001

326 kg

wait for STABLE

Auto      Auto

NO.1

0001

326 kg

after the preset delay

Auto         Auto

NO.1

0001

326 kg

b  0005

*Note: “b 005” means that the current saving is for weighing load No. 5.

printer prints this result

Auto                    Auto

Load Weight

  - Add (below description is effective only without using HEADER key):

    - Manual Add – Press Add key when the system is in manual WEIGH mode (AUTO is not lit in the display), the weighing result will be saved in the internal memory which can be searched and summarized for printing.  Sequence number will increase by step of 1.  For the same stable weigh value, it can only be saved once.

Load Weight

NO.1

0001

0kg

NO.1

0001

326 kg

NO.1

0001

326kg

b  0005

Press ADD

    - Auto Add – Press Auto key to activate Auto Add for data saving and printing.  The AUTO indicator has to be lit on the display.  Under automatic operation, each non-zero stable measurement will be saved and printed automatically.  However, the indicator has to be zeroed between measurements or 50 division numbers.

  - Total

In WEIGH mode, press Total key and the Total sign will light on the display.  Under Total, “b 0007” is shown, for example.  “b” indicates weighing and “0007” indicates the total weighing number meaning there are 7 set weighing data saved so far.  Press Total again, and the display shows “H100kg”, for example.  “H” indicates the data’s high digit, “100” indicates that the total weight high digit is 100.  Press Total again, the display shows “L2837.0kg”.  “L” means lower digits of total weight as “2837kg”.  So the total weight should be 1,002,837kg.  Press Off/Cancel to exit total weight display.

NO.1

0001

12kg

*

b  0007

Press TOTAL

*

H  100 kg

Press TOTAL

*

L2837.0 kg

Press TOTAL

NO.10001

12kg

Press OFF/CANCEL

  - Print Header

In WEIGH mode, press Print H will print the weighing results in the following format:

Weighting list

2004 /08/12 16：00

No.2:0005

Unit：Kg

Press PRINT HEAEDER

Press this key can also perform statistic print function.  In the above example, when the indicator shows a stable weight measurement, press Print.  The printer will print the following results:

Press Print

b             28

“0003” is the current sequence number and “2014” is the current load weight.  Both numbers will be saved in the data memory.  Sequentially, the following data set will be saved and printed with sequence number “0004”, “0005”, “0006”… Upon weighing completion, press Add, the indicator will summarize and print the total weight by the total weighing numbers, starting from Print H:

Weighting list

2003 /08/12 16：00

NO.2:   0005

Unit:     Kg

0003     2000.0

0004     1500.0

0005     1000.0

0006     500.0

------------------------COUNT：4

Total：     5000.0

Press Total

Note: The scale needs to be zeroed between each weighing operation.

  - Order

When Order is pressed, Index is highlighted with the current indexing number, such as “Order 1234”.  If the order number needs to be modified, use the numerical keys (0~9) to input and then press Backlight/Confirm to confirm changes or press Off/Cancel to cancel the operation.  In the following example, the order number is changed from 1234 to 0018.

The modification range of order number is from 0000 ~ 9998.  In case the input order number is 9999 and Backlight/Confirm is pressed, the indicator will show “52909”.  “52909” is actually the current received A/D value.  Press Off/Cancel to return to WEIGH mode of operation.

NO.1

0001

0kg

NO.2

1234

Press NO.2

NO.2

0018

Input NO. 0018

NO.1

0001

0kg

Press Backlight/Confirm

  - Division

Press DIV key to display the current division number.  Use numerical keys 0~9 to select desired division number.  Press Backlight/Confirm to confirm or Off/Cancel to cancel.  The correspondence between numerical number and division number are as following:

1 – 0.1

2 – 0.2

3 – 0.5

4 – 1.0

5 – 2.0

6 – 5.0

7 – 10.0

8 – 20.0

9 – 50.0

The following example is to change the division value from 0.2 to 2.0.

NO.1

0001

12．8kg

Press Division

d      0．2

d      2．0

Press5

NO.1

0001

12kg

Press Backlight/Confirm

  - TARE

In TARE mode, use numerical keys to enter the tare value then press Backlight/Confirm.  The weighing results afterward will be net weights.

NO.1

0001

678.9kg

p               kg

0678.9kg

Press Tare

NO.1

0001

444.4kg

p               kg

0234.5kg

Press Backlight/Confirm

In this example, 234.5kg is the tare and 444.4kg is the net.

When TARE key is pressed, the indicator will display P XXXX.Xkg.  This is the current Tare weight.  If there is no tare weight defined, press TARE key, XXXX.X is the current weight value.  Press Backlight/Confirm to define the weight as tare.  TARE indicator is lit while the net weight is displayed.  With tare defined, the display will show the net weight of the load. Press TARE again to change XXXX.X to 0.0000.  Now press Backlight/Conform to show the gross weight (including tare):

NO.1

0001

678.9kg

p               kg

0678.9

T

NO.1

0001

0.0kg

T

NO.1

0001

333.3kg

Load Weight

p               kg

0000.0   kg

NO.1

0001

1012.2kg

Press Tare

Press Backlight/Confirm

Press Backlight/Confirm

Press Tare

  - Subtract

Subtract function is specially designed for steel milling or molding operation.  Under the normal weighing operation for example, the weight of a load is 567.8kg.  Now enter the Subtract mode of operation by pressing Subtract so the displayed weight value becomes “0.0 kg”. When a 50kg of load is removed, the indicator will show the 50kg as subtraction.  Press Subtract again, the left weight is shown (as normal weight).  The diagram below shows an example of Subtract operation.

NO.1

0001

567.8kg

NO.1

0001

0.0kg

S

NO.1

0001

50.0kg

Discharge 50kg of  goods

NO.1

0001

517.8kg

Repress Subtract

567.8kg

Press Subtract

  - Feed

When Feed key is pressed, the paper will be forwarded 4 lines without printing.

  - Search

When Search key is pressed, the display will show the weighing result prior to the current sequence number.  Press Backlight/Confirm key, the indicator will show the weight result for the previous sequence until Sequence number 0001.  If one keeps pressing Backlight/Confirm key, “nod---”　will be displayed, indicating all the weighing data have been displayed already.  Press Off/Cancel to return WEIGH mode.  In addition, one can search any weighing result by entering the four-digit sequence number to show its weight record.  Press Backlight/Confirm to continue to the previous sequence until the end.  In case there is no weight record for the sequence entered, the display will show “Nod---”.  Press Off/Cancel to return to WEIGH mode.

NO.1

0008

0.0kg

NO.1

0007

234.8kg

NO.1

0006

234.8kg

Press Backlight/Confirm

NO.1

0002

890.0kg

Press Backlight/Confirm after input 0002

NO.1

0001

356.5kg

Press Backlight/Confirm

nod---

Press Backlight/Confirm

NO.1

0008

0.0kg

Press Search

  - Backlight/Confirm

In WEIGH mode, press Backlight/Confirm key to turn on the backlighting.  Press it again will turn the backlight off.  Under other operating conditions, the key is used as confirmation.  If there is no weighing activity 2 minutes after the backlight is turned on, the indicator will turn the light off to preserve battery power.

  - SET

Press SET key to enter setup mode of operation.  “SET” is displayed.  Press numerical key to select the desired function.  The numerical key represents the function in SET mode as below:

NO.1

0008

0.0kg

SEt

A

1 –	Clear

When 1 is pressed under SET mode, the last entry (the weight record with highest sequence number) will be displayed.  A few seconds after, “dEC---“ is showing.  Press Backlight/Confirm to clear or Off/Cancel to cancel.

A

Press 1

NO.1

0002

890.0kg

dEC---

After a few seconds

A

Press Backlight/Confirm or OFF/CANCEL

If there is no stored weight data, the display will show “nod---”.  Press Off/Cancel to return to WEIGH mode.

2 –	Summarize

Press 2 key under SET to enter Print Summery mode (or PRB).  In the Prb menu, there are three options, PRB1, PRB2 and PRB3:

PRB1 means to print summary according to sequence number.

PRB2 means to print summary according to order number.

PRB3 means to print summery according to weighing date.

Input the desired sequence number, order and date for the summary after selecting the summary method.  One can go direct printing by entering the sequence, order or date data.  SUCC will be displayed upon printing completion and FAIL indicates otherwise. Regardless SUCC or FAIL, press Backlight/Confirm to return to input mode to continue. Press Off/Cancel to return to PRB mode.

    - Printing per Sequence number

A

Press 2

Prb

NO.1

0001

NO.1

0021

Input 0021

Press Backlight/Confirm and print the result

SUCC

NO.1

0001

Press1

Press Backlight/Confirm

Weighting list

2005/07/19 14：42

Unit：kg

0001  500.0

The result is below：

    - Printing per Order Number

Press 2

Prb

NO.2

0001

NO.2

1234

Press Backlight/Confirm and print the result

SUCC

Press 2

Press OFF/Cancel

A

Input 1234

Prb

The result is below:

Weighting list

2005/07/19 14：55

NO.2：1234

Unit：kg

0002     1500.0

0003     2999.0

------------------------

COUNT ： 0002

Total： 4499.0

    - Printing per Weighing Date (insert Diagram)

Press 2

Prb

Date

000001

Date

050719

Press Backlight/Confirm and print the result

SUCC

Press 3

Press OFF/Cancel

Input 050719

Prb

The result is below:

Weighting list

2005/07/19

Unit：kg

0001     500.0

0002     1500.0

0003     2999.0

0009     1000.0

------------------------

COUNT ： 0004

Total ： 5999.0

A

3 –	Total Clear

Press 3 under SET menu, the total number of weighing and the total weight are shown first.  In the below example, “0003” indicates the total number of weighing and “4990.0” is the total weight.  After about 2 seconds, the “Clr---“ comes up.  Press Backlight/Confirm to clear all the existing data in the memory.  Press Off/Cancel to quite.  After clearing, the current sequence becomes 0001.

A

0003

4990.0kg

Clr---

After a few seconds

A

Press Backlight/Confirm or OFF/Cancel

Press3

4 –	Set Date

Press 4 to show the current date: i.e.05.02.18 (mm/dd/yr)

Please follow the below example for date change.

A

A

Date

041124

Press4

Date

050719

Input date 050719

Press Backlight/Confirm

5 -	Set Time

Press 5 key to show the current time: i.e. 08:48:21

A

081156

Press 5按5

150103

Input date 150103

Press Backlight/Confirm

A

6-	Zero Range

Press 6 key under SET menu, “O 100” is shown.  “O” is the high digit which indicates the initial zero range at power up.  The low digit indicates the exact zero range (%FS).  For example, “10” means that any drift value which is less than 10% FS will be directly zeroed.  The selections of zero range at power up are as below:

1 –	2%

2 –	10%

3 –	20%

4 –	50%

5 -100%

In the example below, the zero range is changed from 20% FS to 100%FS:

A

o

20

Press6

Press5

Press Backlight/Confirm

A

o

100

0 – Others to enter submenu (password required)

Press 0 key to enter submenu.  After “PAS” prompt, enter the correct password then press Backlight/Confirm.  SS indicates the entry of secondary menu.

A

Press0

PAS

SS

Press Backlight/Confirm after input password

B

There are below selections:

★ Set Weight Limit

After Select 1 in Subset menu, “SSU1 5018kg” is displayed, for example.  Here “SSU1” indicates that the 1st item in the Subset menu is selected and “5518” is the weight capacity limit of the scale.  To modify this number, input the new value and press Backlight/Confirm to finish.  Otherwise Off/Cancel to quit.

In the below example, the weight capacity limit is changed from 3009kg to 5018kg.

B

SSU1

3009kg

Press 1

SSU1

5018kg

Input 5018

Press Backlight/Confirm

B

★ Calibration

When 2 key is pressed under Subset menu, “noloAd” is shown, indicating the system is in calibration mode.  The calibration procedure is as below:

Linear Calibration:

Removal any load from the digital scale.  After the system is in calibration mode and “noLoAd” is shown, press Backlight/Confirm to start calibration.  The display will show “XXXXXXX” which is the internal code for zero.  After the code is stable and “Load1” is shown, the display will go blank within 2 seconds.  Now enter the load weight value.  Press Backlight/Confirm.  The display will show “YYYYYY” which is the internal code for the load weight #1.  After the code display becomes stable, press Off/Cancel to complete the one-time linear calibration between 0 and Load1.  If Backlight/Confirm is pressed, the system is going to non-linear calibration.  The following example is the linear calibration process for a 5-ton crane scale:

B

Press 2

noloAd

Press Backlight/Confirm with non-load

确认

5245

Press Backlight/Confirm

LoAd1

After a few seconds

SSU1

5000

SSU1

Load 5-ton weight and input 5000

Press Backlight/Confirm

591144

Press Backlight/Confirm

LoAd2

B

Press OFF/Cancel

Non-linear Calibration:

Continuing from the linear calibration, press Backlight/Confirm key when “Load2” is displayed.  Now the weight value of Load2 should be entered, and then press Backlight/Confirm key.  Now the “ZZZZZZ” will show.  ZZZZZZ is the internal code for Load 2.  After the internal code becomes stable, the display will show “Load3”.  If a three-point nonlinear calibration (0, Load 1 and Load 2) is desired, press Off/Cancel to complete.  Otherwise press Backlight/Confirm.  When the LCD display goes blank, enter the weight value for Load3 and then press Backlight/Cancel.  The internal code for Load3 is showing.  After the display becomes stable, press Backlight/Confirm to complete the four-point nonlinear calibration.

Below is an operating example that a 20-ton crane scale is non-linear calibrated with 3-ton, 8ton and 20-ton weight.

B

Press 2

noloAd

Press Backlight/Confirm with non-load

5245

Press Backlight/Confirm

LoAd1

A few seconds later

Lod1

3000

Lod1

Load 3-ton weight and input 3000

Press Backlight/Confirm

151416

Press Backlight/Confirm

LoAd2

Press Backlight/Confirm

LoAd1

Lod1

8000

Lod2

Load 8-ton weight and input 8000

Press Backlight/Confirm

257279

LoAd2

Press Backlight/Confirm

LoAd3

Lod3

Press Backlight/Confirm

★ Change Password

In Subset Menu pressing 3 key, enter the new password when “PAS1” is shown.  Press Backlight/Cancel upon completion.  Enter the new password one more time when the display is showing “PAS2” and press Backlight/Confirm.  If both passwords entered are identical, the password modification is complete.  Otherwise return to Subset Menu and repeat the above process to complete the password modification.

Below example is showing the procedure to change the password to “123456”.

B

B

PAS1

Press 3

123456

PAS1

Enter the new password 123456

PAS2

Re-enter the new password

123456

PAS2

Press Backlight/Confirm

Press Backlight/Confirm

.

★ Zero-point Track Range

When 4 key is pressed under Subset menu, TO 1 is displayed.  “TO” indicates the range of zero point tracking and the digit “1” indicates the detail of this range.  There are several selections with their correspondence with the numerical keys.

1 –	Off

2 –	Small

3 –	Moderate

4 –	Large

The below example is to change the tracking range from Off to Small.

B

Press 4

1

TO

2

TO

B

Press Backlight/Confirm

Press 2

★ Filter Setup

Press 5 key in Subset menu to enter Filter Setup.  The display, for example, is “F 30”.  “F 30” is the extent of filtering.  If no filtering change is needed, press Off/Cancel to quit.  Otherwise press Backlight/Cancel to proceed to Filter Setup.  In the example below, the filter is set from 30 to 40.

30

F

40

F

Press 5

Press40

B

Press Backlight/Confirm

★Auto Print Setup

In Subset menu, press 6 key.  The display is “A1 05.2”, for example.  Here “05.2” indicates that the auto-printing will start 5.2 seconds after the weighing completion (or the weight measured becomes stable).  If no modification is needed, press Backlight/Confirm otherwise, enter the desired time (0.1~99.9) and press Backlight/Confirm to complete.  Now, “A2 0.5” will be shown.  Here “0.5” means the data variation range in division, within which it is considered stable during measurement.  In this case, the stable range is 0.5 of one division.  If no change, Backlight/Confirm to quit.  Otherwise enter the new value (0.1~999.9).  Press Backlight/Confirm to complete.

In the following example, the auto-print time delay is changed from 5.2 seconds to 3 seconds and the stable range is from 0.5d to 1d.

05.2

A1

03.0

A1

Press 6

Enter030

B

Press Backlight/Confirm

000.5

A2

001．0

A2

Enter 0010

Press Backlight/Confirm

★Printer Setup

In Subset menu, press 7 key.  The display shows “PS 1”.  “PS” is the printer setup and “1” indicates that there is a printer (“0” means there is no printer).  Enter “0” to skip printing or enter “1” to select printing.  Press Backlight/Cancel to return to Subset menu.

B

0

PS

1

PS

Press 1

Press 7

Press Backlight/Confirm

B

★ Indicator Exchange

Press 8 key in Subset Menu and the indicator shows “XXXXXX”.  XXXXXX is the calibration factor.  If there is no change, press Backlight/Confirm.  Otherwise enter the new factor then Backlight/Confirm.  Now the display shows “E  X”. X can be 0, 1 or 2 with the following implications:

When X=0: if XXXXXX is 123456, then the calibration factor is 123456.

When X=1: if XXXXXX is 123456, then the calibration factor is 1234560.

So X is the power of 10 for the calibration factor.

Press Backlight/Confirm to complete the indicator exchange process.

In the example below, “928637” is the calibration factor.  In the example, the calibration factor is changed to 1021500.  Since the ratio between 1021500/928637 =110%, the change will result in weight display change of more than 10% from the old factor.  Since the LCD display can only display 5 digits, so only 102150 is entered but change the E value to 1 (10x).

B

928637

Press 8

102150

Enter 102150

Press Backlight/Confirm

E    0

E    1

Press 1

- Battery Charging Method

Both scale and indicator are powered by the batteries.  For the scale, the battery can be charged either while it is installed on-scale or it can be taken out of scale and charged.  Battery should be charged promptly once the indicator power monitor shows the low battery power.  This is to prolong the battery life without over discharge.  The power supply for the charger is 220V.  It will take over 5 hours to fully charge the battery (depending on the battery power level at the time of charge).

- Precautions

Digital crane scales are precision measurement equipments.  Proper maintenance is important to prolong their usable lifetime.  Please note the following:

    - Read carefully the user guide before operating the scale and the indicator.

    - Do not overload the scale.

    - The crane motion should be up/down in vertical fashion.  Do not drag or carry the weight load through the scale.

    - The scale can not be used extensively under high ambient temperature.  Even with thermal isolation apparatus, effort should be made to minimize the scale’s heat exposure.  A rule of thumb is to the scale body is cool enough to touch (or below 70° C) so it can operate normally.

    - Avoid hard banging and wafer from the scale.

    - Avoid personal injury by properly hanging of the scale body.

    - Minimize the usage of backlighting to preserve battery power.

    - Nd-H batteries are used for the scale and the indicator.  Do not operate under low charge condition to prolong battery life.  It is recommended that the battery to be disconnected from the scale if the scale is inactive for over 20 hours. Power off the indicator main if it is inactive for over a week.  Charge the batteries each month even without usage.

    - The indicator should be stored under dry and cool environment.

    - Use only the supplied charger and avoid over charging.

    - Handle gently the indicator handle.

    - Keep a safe distance when operating the crane scale from the weight load to prevent personal injury from falling objects.

    - For Subset menu operation, do not attempt to alter the setup without proper training.

    - Please contact us if the scale needs to be calibrated.

- Trouble- shooting

    - Beeper continuous beeps – load weight is exceeding the weight capacity of the scale.  Please reduce the weight of load for normal operation.

    - The indicator shows only the time but no weight number – no signal reception from the scale.  Please check if the A/D convert is properly connected and the battery is properly charged (>6Volts).  If there is no reception within 10 meters, please check if the scale antenna has good contact or if it is damaged.  Use backup antenna if needed.

    - The indicator has no display even LCD screen is lit.  It is either due to a bad or lowly charged battery.  If the problem persists after recharge of the battery, change the battery.

    - If the indicator receives signal from the scale, however the weight display remains at 0 regardless of the weight, please follow the instruction on page ?? .  Enter 9999 and press Backlight/Confirm.  Then pull the scale slightly to see if there is change in internal code.  If there is no change, it indicates that the transducer (or load cell) has bad contact.  Please check the connection wire.

    - During the printing process, if there is only printing sounds without paper out, then there is no more printing paper.  Please reload.  To load the paper, please following these steps:

      - Turn off the power supply and open the printer cover.

      - Feed the paper through the entry slot.  Turn on the power and press Feed.  The printer paper should feed automatically.

      - Close the printer cover.

    - If the indicator displays “ERROR0” at power on and no head printing with Print H, they indicate that the printer for the indicator is broken.  Please contact the vendor for a new printer.

    - In case the printing is illegible, the color ribbon need to be changed:

      - Open the printer cover

      - Carefully remove the old color ribbon.  Do not touch the printer bearings.

      - Insert the new ribbon box gently into the left end of bearing axis with right side slightly up.  If the ribbon box is not reach the bottom, please press the knob on the ribbon box and rotate slightly by following the arrow indicator. Until the left side of ribbon box hits the bottom, low the right end of the box.

      - Check if the ribbon is straight.  If not, please rotate the knob on the left side of ribbon box to straight it out.

      - Reinstall the printer cover.

Please note:

The printer model and its accessories are as below:

- Roll paper: width: 44.5±0.5mm, thickness 0.07mm.  Roll paper OD is <40mm.

- Ribbon: EPSON ERC-05.

- Printer: EPSON-150II.

- After Sale Service

Please feedback any problem encountered during the usage of the scale to our sales representative.  This will ensure us to provide timely technical support and quality service.  Please do not attempt to fix the problem which may void the existing warranty.

- Packaging List

| Item | Name | Quantity |
| --- | --- | --- |
| 1 | OCS-SZ | 1 |
| 2 | Scale | 1 |
| 3 | Instrument Box | 1 |
| 4 | Ribbon (ERC-05) | 1 |
| 5 | Printer Paper | 2 |
| 6 | Indicator Battery Charger | 1 |
| 7 | Scale battery Charger | 1 |
| 8 | User’s Guide | 1 |